# Solar Energy Storage Basics

## Overview
Solar energy storage systems capture electricity produced by photovoltaic panels
and release it later when generation is low. The most common residential option
is a lithium-ion battery bank paired with a hybrid inverter.

## Capacity planning
Battery capacity is measured in kilowatt-hours (kWh). A typical household that
consumes 20 kWh per day may target 10 to 15 kWh of usable storage for evening
coverage. Depth of discharge, round-trip efficiency, and temperature all affect
usable capacity.

## Safety notes
Installations must follow local electrical codes. Battery rooms need ventilation
and thermal monitoring. Never mix incompatible chemistries in the same bank.

## Maintenance
Inspect connections quarterly. Firmware updates for inverters should be applied
only from the manufacturer channel. Replace cells that show abnormal voltage
drift during monthly health checks.